| Fuel Injection > A/C Clutch |  |
| --- | --- |

| Secondary Air Injection | Electric Cooling Fan |
| --- | --- |

A/C Clutch

To improve idle quality and WOT performance, the PCM controls engagement of the A/C clutch on some systems (figure 4-49). - At idle, the PCM first commands the IAC valve to increase engine idle slightly, then grounds the A/C relay. This minimizes poor idle quality or stalling. - The PCM will interrupt A/C operation at wide-open throttle, when maximum engine performance is desired. - When the power steering pressure (PSP) switch signals high power steering pressure at low engine speed (during parking, for instance), the PCM removes the ground from the A/C clutch relay to prevent stalling caused by the compressor load on the engine. - If engine cooling system overheating is indicated by the coolant temperature sensor, A/C operation is interrupted.

Figure 4-49, A/C Clutch Control Circuit